WHALE CALL ANALYSIS OUTPUT FILES Revised for Shell Archival Data

(Put together by Susanna Blackwell and Katherine Kim, 16 Aug. 2011, revised by Alexander Conrad and SB, 12 Nov. 2014; finalized by SB July 2016)

File naming nomenclature:  Files are named AllSiteX_YYYY_auto_archive.txt or AllSiteX_YYYY_manual_archive.txt where X is the site (0–5), YYYY is the year in which the data were collected, and auto versus manual indicates whether the data were analyzed by detection software (auto, see below) or by human analysts (manual).  For example, AllSite2_2009_manual_archive.txt contains all the manually analyzed whale call outputs for site 2 in 2009.

Manual versus automated analysis in 2007–2014:

2007:  the entire season was analyzed both with the manual and automated methods.

Starting in 2008, automated analysis was the main analysis method (the method is described in Thode et al. 2012).  In 2012 an additional processing stage was added to remove false detections generated by industrial sounds, arising from both vessel transits and site drilling.  This additional processing is described in Blackwell and Grebner (2016).  Each year, 6–9 days were analyzed manually to provide training material for the neural network of the automated algorithm, and then to check its performance.  The period midnight to noon of each day analyzed manually was used to train the algorithm, while the period noon to midnight of each day analyzed manually was used to check the performance of the trained algorithm (the training was not done on a yearly basis).  The following days were analyzed manually each year:

2008:  21 and 28 August, 6, 13, 21, and 29 September.

2009:  27 August, 3, 8, 12, 14, 18, 25, and 30 September.

2010:  15, 21, and 29 August, 5, 13, and 27 September.

2011:  2, 9, 18 and 25 August, 1, 9, 14, and 28 September.

2012:  25 August, 1, 7, 13, 18, 23, 26, and 29 September, 5 October.

2013:  10, 18, and 28 August, 4, 20, and 28 September.

2014:  18, and 28 August, 1, 17, and 27 September.

In 2013 the number of calls in the acoustic records was tremendous, and we were forced to reduce the numbers of hours analyzed.  Sites 0, 1, and 2 were analyzed in their entirety, whereas sites 3, 4, and 5 were only partially analyzed.  Midway though the analysis, each hour remaining to be analyzed was split into four quarters (00:00–14:59 min:sec, 15:00–29:59, 30:00–44:59, and 45:00–59:59) and only the first and third quarters were analyzed.  This system was implemented partway through the season, however, so the percentage of each hour that was analyzed can be 50%, 75%, or 100%, and varies by site and hour of the day.

The tab-delimited file 2013_analysis_times.txt includes information on which quarters of every hour were analyzed for the six days analyzed manually in 2013.  It is formatted as follows:

Date		Hour	Q	Site 3		Site 4		Site 5

10-Aug-13	0	1	1		1		1

10-Aug-13	0	2	0		1		0

10-Aug-13	0	3	1		1		1

10-Aug-13	0	4	0		1		0

10-Aug-13	1	1	1		1		1

Hour 0 is midnight to 1 am, hour 1 is 1–2 am, etc.  Q stands for quarter of that hour (see above).  In the three rightmost columns (corresponding to sites 3, 4, and 5), “1” means the quarter was analyzed at that site, and “0” means it was skipped.

Note that in some years, not every day was always analyzed at every site.  For example, it happened that a day was analyzed at the end of the season when one or several sites had already been retrieved.

File organization:

The files are organized as follows:  Each line in the file corresponds to one bowhead whale call.  In the files produced by automated analysis, each call was detected by a minimum of two DASARs and is therefore generally (but not always) successfully localized.  In the files produced by manual analysis all detected calls are listed, even those detected by only one DASAR.  (Therefore, such calls cannot be localized and have no associated location estimate, see below).

Each line has a variable number of columns, separated by tabs.  The number of columns (fields) is dependent on how many DASARs detected that particular call (see below).  The first 25 columns contain data related to the call itself and its location.  There follows a variable number of 12-column groups, each associated with one DASAR on which the call was detected.  For example, if a call was detected on three DASARs, then the line corresponding to that call will have 25 + (3  12) = 61 columns.

The contents of the columns are described below.  Column numbers are marked with an asterisk (*) if the values in that column are replaced with “NaN” (Not a Number) when the call was not successfully located.

Column 1:  Calculated time in AKDT of the whale call expressed in C programming time format, i.e., the number of seconds elapsed since midnight, 1 January 1970. (This format is convenient for manipulating time within software programs.)  The time is calculated by first calculating the distance from one of the DASARs (the first alphabetically) to the estimated position of the call, dividing that range (in meters) by 1440 m/s and subtracting that time delay from the time when that DASAR heard the call.  If the call was not successfully located, the call time is the earliest time of detection at any of the DASARs that detected that call.

Column 2:  The same time as in column 1 (again, in AKDT), expressed as a conventional human-readable date string.  (DD-MMM-YYYY hh:mm:ss)

Column 3:  Calculated time of the whale call in UTC expressed in C programming time format (see Column 1).  The time difference between UTC and AKDT is 8 hours, so the numbers in columns 1 and 3 should be different by 86060 = 28,800 seconds.

Column 4:  The same time as in columns 3 in UTC, expressed as a conventional human-readable date string.  (DD-MMM-YYYY hh:mm:ss)  The times in Columns 3 and 4 should be different from the times in Columns 1 and 2 by exactly 8 hours.

Column 5*:  Calculated UTM (Universal Transverse Mercator) easting coordinate of the estimated location of the whale call.  The location is determined using the Huber robust location estimator, a method of estimating a two-dimensional location based upon observed directional bearings from known positions (Lenth 1981).  Note that the five DASAR sites are located in three different UTM zones:  site 1 is in zone W5, sites 2, 3, 0, and 4 are in zone W6, and site 5 is in zone W7.  For consistency, all calls computed for a given site are expressed in that site’s UTM zone.  For example, site 3 call locations will have eastings expressed relative to zone W6, even though some of the calls may actually reside in other zones to the east or west of zone W6.

Column 6*:  Calculated UTM northing coordinate of the call.

Column 7: UTM Zone of the call.  As mentioned in the description of Column 4, this is not calculated.  It is the same for every call at a given site.

Column 8*:  Calculated latitude of the call in decimal degrees (positive for northern, negative for southern hemisphere).

Column 9*:  Calculated longitude of call in decimal degrees (positive for eastern, negative for western hemisphere).

Column 10*:  Distance (in m) from center of DASAR array.  The center of the DASAR array was calculated for each site, each year, by taking the average easting and northing of each array’s DASARs.  The positions of DASARs 4H and 4I, however, were always excluded from these calculations.

Column 11:  Call type.  Call classification is achieved with manual call analysis (i.e., analysis performed by humans) but not with automated analysis (performed by computer).  As a consequence of this, the automated files contain only bowhead whale calls, whereas the manually analyzed files contain a mix of calls from bowheads and other marine mammals, as well as a few possible man-made sounds.  If the output file results from automated analysis, this column will contain the value “-1”.  Call type categories are shown below.  They changed somewhat between 2007 and following years, but have been standardized to the list below.

| Label | Call type |
| --- | --- |
| 1 | Upcall |
| 2 | Downcall |
| 3 | Near-constant tone (“constant call”) |
| 4 | -shaped undulation |
| 5 | -shaped undulation |
| 6 | Other undulation, mixed undulation |
| 7 | Complex call, high call, growl, |
| 8 | Bearded seal |
| 9 | Walrus |
| 10 | Man-made, calibration tones |
| 11 | Uncertain marine mammal |
| 12 | Unknown, other |
| 13 | Call sequence begin |
| 14 | Call sequence end |

Column 12*:  Area of the call’s 90% confidence ellipse (as determined during call localization using the Huber robust location estimator, see Lenth 1981), in m2.

Column 13*:  Length in meters of the major axis of the 90% confidence ellipse.

Column 14*:  Length in meters of the minor axis of the 90% confidence ellipse.

Column 15*:  Bearing in degrees of the semi-major axis of the 90% confidence ellipse.  In files produced from the automated analysis these values are in radians.

Column 16*:  Angle in radians counterclockwise of the semi-major axis, referenced to an imaginary vector emanating from the center of the DASAR array and directed toward the estimated location.

Column 17:  Two values written in the format “# (#)” to indicate how many DASARs were used to estimate call location.  In files produced by automated analysis, the first number will not be less than 2.  In files produced by manual analysis, the first number can be as low as 1, indicating a call was detected by only one DASAR and, therefore, not localized.  The second number in parentheses is the number of DASARs given a non-zero weight and being included in the location estimation.  A weight of zero for a particular DASAR occurs for one of two reasons:  (1) if the bearing was far off of the other units and the Von Mise estimator gave it a very small weight that gets rounded to zero (first four significant digits are 0), or (2) if bearings of a particular unit were known to be bad and the weight of its bearings for all of its calls were set to zero so it would not be counted (more description of this below).  If no successful solution to the call localization is achieved, then all units are given weights of zero and the number in parentheses is 0.

Columns 18*, 19*, 20*, and 21*:  The four elements comprising the covariance matrix associated with the Von Mise maximum likelihood estimate of call location.

Column 22:  ASCII text (character string) identifying whether a good solution for the location estimate was found and, if not, the reason for failure.

Column 23:  Operator comment.  In output files from automated analysis, this column will contain the word “automated”.  Manual analyst comments may include “UN-Xed:_XX” for example, UN-Xed:_AC) where XX are one or several DASAR letters indicating that, given the opportunity, the analyst would remove the bearing from that (those) DASARs because although it heard the call, the resulting bearing didn’t match what the other DASARs heard.  Nevertheless, to keep results unbiased, the localization algorithm did not allow analysts to remove bearings.  Instead the bearings were weighted based on quantifiable factors.  The analyst’s preference is only preserved here in the comments.

Column 24:  Operator ID.  In output files from manual analysis, this column will contain the initials of the human analyst.  In output files from automated analysis, this column contains “0”.

Column 25:  Date and time of processing (in UTC), i.e., date and time at which the call was identified and analyzed by human or computer methods.  Many calls that were re-localized long after they were first marked will have a different date in Column 24 than calls occurring before or after (above or below) in the records.

Following these 25 columns are groups of 12 columns (columns A–L), each group corresponding to one of the DASARs that heard the call in question.  For example, a call that was detected by DASARs A and D at a particular site will have 25 + (2  12) = 49 columns, with 12 columns relating to the detection by DASAR A and 12 columns relating to the detection by DASAR D.  The first column of each group of 12 (column A, see below) contains the ID of the DASAR to which all the information in the 12 columns pertains.  Note that not all the DASARs that detected a call will necessarily be used equally to calculate the localization estimate (see Column B below).

Column A:  DASAR ID, in the format SXYYWZ, where X is the site (0, 1, 2, 3, 4, or 5), YY is the two digit year, and W is the DASAR location within an array, and Z is the deployment ID number, which is generally 0.  When it is not, it indicates that a DASAR was redeployed, for example after failing a health check.  For example, S408G0 is the first deployment of DASAR G at site 4 in 2008 (see Figure 1 below, showing DASAR naming convention within arrays).

In 2012–2014 site 4 was “flipped”, with positions B, D, and F maintaining their positions and positions A, C, E, and G getting moved ~12.5 km to the east.  For these new positions the value of Z was set to 1.  So in 2012, the easternmost DASARs at site 4 therefore had the IDs S412A1, S412C1, S412E1 and so on.

Column B:  Weight assigned to whale call bearing in location estimate from a given DASAR.  Between 0 and 1.  If a solution position was not found, the weights of all DASARs are marked as 0.

Column C:  brel, i.e., whale call bearing in degrees measured relative to the DASAR.

Column D:  bgrid, i.e., whale call bearing in degrees relative to True North.   For example, if bgrid = 0°, then the call originated due north of the DASAR.  This takes into account bref, which is the orientation of the DASAR on the seafloor relative to True North.

Column E:  Measured arrival time (UTC) of call at the DASAR, in C programming time format (see Column 3 above).  The estimated precision is about 1 second, equivalent to about 1500 meters distance.  This arrival time is not used in the location calculation.

Column F:  Root-mean-square (rms) level, in dB re 1 Pa, of call received on DASAR’s omnidirectional hydrophone, calculated over the measured band and measured length of the call.  The background noise, measured for one second starting two seconds before the call, is subtracted to yield this estimate of signal-without-noise call levels.  This process is subject to random errors because of the variability of the background noise with time.

Column G:  Signal-to-noise ratio (SNR) in the band of the call, expressed in dB.

Column H:  Lowest frequency of call, in Hz, as defined by a human operator during manual analysis or by the computer algorithm during automated analysis.  Note that this value does not always correspond to the actual lowest frequency of the call, because in conditions of high background levels, the manual analysts may select the higher-amplitude parts of the call to get a better bearing accuracy.

Column I:  Highest frequency of call, in Hz, as defined by a human operator during manual analysis or by the computer algorithm during automated analysis.  Note that this value does not always correspond to the actual highest frequency of the call, because in conditions of high background levels, the manual analysts may select the higher-amplitude parts of the call to get a better bearing accuracy.

Column J:  Length of call, in seconds, as defined by a human operator during manual analysis or by the computer algorithm during automated analysis.  Note that this value does not always correspond to the actual length of the call, because in conditions of high background levels, the manual analysts may select the higher-amplitude parts of the call to get a better bearing accuracy.

Column K:  Standard error estimate of the call bearing (in degrees).

Column L:  Kappa, i.e., Von Mises concentration parameter, another form of the standard error estimate of the call bearing.

Figure 1:  DASAR array patterns.  (a)  The normal configuration includes seven DASARs placed at the vertices of five adjacent equilateral triangles, with 7 km spacing between the recorders.  (b)  In 2010 and 2011, two DASARs were added to the western side of site 4.  (c)  In 2012–2014 site 1 was reduced to three of the original seven DASARs.  (d)  In 2008, site 1 was augmented with five DASARs (H–L) to the southeast of the normal array configuration.  (e)  Ice prevented normal deployment of DASARs at site 2 in 2010, so only DASARs F and G were deployed in their usual location.  (f)  In 2012–2014 the westernmost DASARs of site 4 (A, C, E, G) were moved ~12.5 km to the east (A1, C1, E1, G1), and four DASARs (J–M) were added to the northern end.  The positions of DASARs H and I were the same as in 2010 and 2011.  (g)  Site 0 was used in 2012 and 2013, with three DASARs separated by 2 km.  Site 0 is located between site 3 and site 4.

References:

Blackwell, S.B., and D.M. Grebner.  2016.  Beaufort Sea Acoustic Monitoring Program, 2012.  Greeneridge Report 479-2.  Report from Greeneridge Sciences, Inc. (Santa Barbara, CA) for Shell Offshore, Inc. (Anchorage, AK).  30 p.

Lenth, R. V.  1981.  On finding the source of a signal.  Technometrics 23:149–154.

Thode, A.M., K.H. Kim, S.B. Blackwell, C.R. Greene, Jr., C.S. Nations, T.L. McDonald, and A.M. Macrander.  2012.  Automated detection and localization of bowhead whale sounds in the presence of seismic airgun surveys.  Journal of the Acoustical Society of America 131(5):3726–3747.